以下为您提供纯英文版本的专业翻译（已与中文完全分开，方便您直接复制使用）：

### 1. Identification of the Substance/Mixture and of the Company/Undertaking

* **1.1 Product Identifier:**

* **Product Name:** Waterborne Polyurethane Dispersion PU-608

* **Chemical Classification:** Aqueous dispersion of modified polyurethane resin

* **1.2 Relevant Identified Uses of the Substance or Mixture and Uses Advised Against:**Industrial applications in metal coatings, porous flooring, paper coatings, wood coatings, and leather finishing.

* **1.3 Details of the Supplier of the Safety Data Sheet:**

* **Supplier Name:** Guangzhou Guanzhi New Material Technology Co., Ltd.

* **Supplier Address:** Room 1106, Area A, Guangzhou International Business Incubator, No. 3 Juquan Road, Science City, Luogang District, Guangzhou

* **Telephone:** 86-20-82567990

* **Fax:** 86-20-32214789

---

### 2. Hazards Identification

* **2.1 GHS Classification:** Reproductive toxicity, Category 1B (H360)

* **2.2 GHS Label Elements:**

* **GHS Pictogram:** (Health Hazard Symbol)

* **2.3 Signal Word:** Danger

* **2.4 Hazard Statements:** H360 May damage the unborn child.

* **2.5 Precautionary Statements:**

* **Prevention:**

* P201 Obtain special instructions before use.

* P202 Do not handle until all safety precautions have been read and understood.

* P280 Wear protective gloves/protective clothing/eye protection/face protection.

* **Response:**

* P308+P313 IF exposed or concerned: Get medical advice/attention.

* **Storage:**

* P405 Store locked up.

* **Disposal:**

* P501 Dispose of contents/container in accordance with local regulations.

* **2.6 Other Hazards:** Risk of cutaneous (skin) absorption of N-methyl-2-pyrrolidone.

---

### 3. Composition/Information on Ingredients

* **Product Type:** Mixture

| Components / Chemical Name | CAS No. | Content % (w/w) |

| --- | --- | --- |

| Polyurethane dispersion

| 9009-54-5

| 33.0~36.0

|

| Deionized water

| 7732-18-5

| 50.0~54.0

|

| N-Methyl-2-pyrrolidone (NMP)

| 872-50-4

| 13.0~14.0

|

---

### 4. First Aid Measures

* **4.1 General Advice:** Immediately remove all contaminated clothing.

* **4.2 Ingestion:** In case of accidental ingestion, do not induce vomiting and seek medical advice immediately. Never give anything by mouth to an unconscious person.

* **4.3 Eye Contact:** Immediately hold eyelids open and flush eyes with plenty of gentle running water for at least 20 minutes. Tilt the head to prevent the chemical from flowing into the uncontaminated eye, and seek medical attention immediately.

* **4.4 Skin Contact:** Wash immediately with soap and plenty of water. If a skin reaction occurs, seek medical advice.

* **4.5 Inhalation:** If inhaled and discomfort occurs, seek medical attention.

---

### 5. Firefighting Measures

* **5.1 Suitable Extinguishing Media:** Carbon dioxide (CO2), alcohol-resistant foam, dry chemical powder; use water spray in case of large fires.

* **5.2 Unsuitable Extinguishing Media:** High-volume water jet.

* **5.3 Special Hazards Arising from the Substance or Mixture:** In case of fire, carbon monoxide, carbon dioxide, nitrogen oxides (NOx), and traces of hydrogen cyanide may be released. In case of fire or explosion, do not breathe fumes.

* **5.4 Advice for Firefighters & Protective Equipment:** Firefighters must wear self-contained breathing apparatus (SCBA). Do not allow contaminated fire-extinguishing water to enter the soil, groundwater, or surface water.

---

### 6. Accidental Release Measures

* **6.1 Personal Precautions, Protective Equipment and Emergency Procedures:** Wear personal protective equipment. Ensure adequate ventilation/exhaust. Keep unauthorized personnel away.

* **6.2 Environmental Precautions:** Do not discharge into drains/sewers, wastewater, or soil.

* **6.3 Methods and Material for Containment and Cleaning Up:** Take up with chemical-absorbent material or, if necessary, dry sand, and store in closed containers for disposal.

---

### 7. Handling and Storage

* **Precautions for Safe Handling:** Use only in areas provided with appropriate exhaust ventilation. Avoid contact with skin and eyes. Do not breathe vapors or spray mist. Keep away from food, drink, and tobacco. Wash hands before breaks and at the end of work. Change contaminated or soaked clothing.

* **Conditions for Safe Storage:** Keep container tightly closed in a cool, well-ventilated place. To maintain product quality, observe the storage conditions specified in our Technical Data Sheet (TDS).

---

### 8. Exposure Controls/Personal Protection

* **8.1 Exposure Controls:**

* **Respiratory Protection:** Respiratory protection is required during spraying applications.

* **Hand Protection:** Protective gloves are recommended.

* **Suitable Material for Protective Gloves:** EN 374-3

* **Fluorocarbon rubber – FKM:** Thickness ≥ 0.4 mm; breakthrough time ≥ 480 min.

* **Butyl rubber – IIR:** Thickness ≥ 0.5 mm; breakthrough time ≥ 480 min.

* **Nitrile rubber – NBR:** Thickness ≥ 0.35 mm; breakthrough time ≥ 480 min.

* **Recommendation:** Contaminated gloves should be discarded.

* **Eye Protection:** Wear safety goggles/face shield.

* **Body Protection:** Wear suitable protective clothing.

---

### 9. Physical and Chemical Properties

* **9.1 Appearance:** Translucent emulsion

* **9.2 Odor Threshold:** No data available.

* **9.3 pH (1% aqueous solution):** 7–9

* **9.4 Ionicity:** Anionic

* **9.5 Initial Boiling Point:** Approx. 100°C

* **9.6 Flash Point:** Not applicable

* **9.7 Evaporation Rate:** No data available.

* **9.8 Flammability (solid, gas):** Not applicable

* **9.9 Heat of Combustion:** Not applicable

* **9.10 Vapor Pressure:** Approx. 130 hPa at 50°C

* **9.11 Relative Vapor Density:** No data available.

* **9.12 Density:** 1.0–1.1 g/cm³

* **9.13 Water Solubility:** Completely miscible

* **9.14 Surface Tension:** No data available.

* **9.15 Partition Coefficient (n-octanol/water, log value):** No data available.

* **9.16 Auto-ignition Temperature:** Not applicable

* **9.17 Ignition Temperature:** No data available.

* **9.18 Decomposition Temperature:** No data available.

* **9.19 Dynamic Viscosity:** < 250 mPa·s

* **9.20 Explosive Properties:** No data available.

* **9.21 Dust Explosion Class:** Not applicable

* **9.22 Solids Content (150°C, 30 min):** 35 ± 2%

* **9.23 Other Information:** The above data are not product specifications. For product specifications, please refer to the Technical Data Sheet (TDS).

---

### 10. Stability and Reactivity

* **10.1 Chemical Stability:** No decomposition if used according to specifications.

* **10.2 Hazardous Decomposition Products:** Neutralizing agents are released during drying/curing of the coating.

* **10.3 Possibility of Hazardous Reactions:** No hazardous reactions when stored and handled properly.

---

### 11. Toxicological Information

* No toxicological studies are available for this product.

* The following are toxicological data for N-Methyl-2-pyrrolidone:

* **11.1 Acute Toxicity:**

* **Oral:** LD50 (rat): 4,150 mg/kg

* **Inhalation:** LC50 (1 h, rat): > 5.1 mg/l; LC50 (4 h, rat): > 5.1 mg/l

* **Dermal:** LD50 (rat): > 5,000 mg/kg

* **11.2 Primary Skin Irritation:** Species: Rabbit; Classification: Skin irritant; Result: Causes skin irritation

* **11.3 Primary Mucous Membrane Irritation:** Species: Rabbit; Classification: Eye irritant; Result: Causes serious eye irritation

* **11.4 Sensitization:** Species: Mouse; Classification: Not a skin sensitizer; Result: Did not cause sensitization in laboratory animals

* **11.5 Mutagenicity:** No mutagenic effects observed in bacterial or mammalian cell cultures. No mutagenic effects observed in animal studies.

* **11.6 Carcinogenicity:** No carcinogenic effects observed in animal studies.

* **11.7 Reproductive Toxicity:** In animal studies, a risk of impaired fertility was found only after administration of high doses of this substance.

* **Teratogenicity:** Teratogenic effects were observed in animal studies. May damage the unborn child.

* **11.8 Specific Target Organ Toxicity (Single Exposure / Repeated Exposure):** May cause respiratory tract irritation.

* **11.9 Aspiration Hazard:** No data available.

* **11.10 Additional Information:** N-Methyl-2-pyrrolidone poses a risk of dermal absorption.

---

### 12. Ecological Information

* No ecotoxicological studies are available for this product.

* The following are ecotoxicological data for N-Methyl-2-pyrrolidone:

* **12.1 Ecotoxicity:**

* **Toxicity to Fish:** LC50 (96 h, Oncorhynchus mykiss [rainbow trout]): 500 mg/l

* **Other:** EC50 (72 h, algae): > 500 mg/l; EC50 (24 h, Daphnia magna): > 1,000 mg/l

* **12.2 Persistence and Degradability:** Biodegradability: 73%, 28 d, i.e., readily biodegradable.

* **12.3 Other Adverse Effects:** No data available.

---

### 13. Disposal Considerations

* Disposal must be carried out in accordance with applicable national standards, national laws, or local regulations. Within the EU, disposal should comply with the appropriate regulations of the European Waste Catalogue (EWC).

* **Disposal Methods:** Empty containers as thoroughly as possible (e.g., by pouring, scraping, or draining until "drip-dry") and send them to an appropriate collection point in accordance with existing recycling schemes of the chemical industry. Containers should be recycled in compliance with national laws and environmental regulations. Do not discharge waste into wastewater.

---

### 14. Transport Information

* **14.1 Land Transport (Road and Rail / ADR/RID):** Not classified as dangerous goods for land transport.

* **14.2 Sea Transport (IMDG):** Not classified as dangerous goods for sea transport.

* **14.3 Air Transport (ICAO-TI / IATA-DGR):** Not classified as dangerous goods for air transport.

* **14.4 Special Precautions for User:** Not dangerous goods. Avoid temperatures above +35°C and keep from falling below +5°C. Keep away from food, acids, and alkalis.

---

### 15. Regulatory Information

* **Safety, Health and Environmental Regulations/Legislation Specific for the Substance or Mixture**

* **Other Regulations:** Complies with the requirements of the following regulations:

* *Regulations on the Control over Safety of Dangerous Chemicals* (State Council Decree No. 591)

* *GB/T 16483 Safety Data Sheet for Chemical Products — Content and Order of Sections*

* *GB 13690 General Rule for Classification and Hazard Communication of Chemicals*

* *GB 30000.2-29 Rules for Classification and Labelling of Chemicals*

* *GB 15258 General Rules for Preparation of Precautionary Label for Chemicals*

---

### 16. Other Information

* To the best of our knowledge and belief, the information provided in this Safety Data Sheet is accurate as of the date of its publication.

* The information provided is intended only as a guide for safe handling, use, processing, storage, transportation, disposal, and release, and is not to be considered a warranty or quality specification.

* The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any other process, unless specified in the text.