| - Identification |  |
| --- | --- |
| - 1.1 Product name： | Cationic polyurethane dispersion PU-2702 |
| Chemical category： | Polyurethane dispersion |
| - 1.2 Product use suggestions and restrictions： | Coatings and inks |
| - 1.3 Supplier information: |  |
| Name of supplier： | Guangzhou Guanzhi New Materials Technology Co., Ltd. |
| Supplier address: | Room 1106, Area A, Guangzhou International Enterprise Incubator, No. 3 Juquan Road, Science City, Huangpu District, Guangzhou |
| Tel： | 86-20-82567990 |
| Fax： | 86-20-32214789 |

- 2.Hazards Identification

Classification of the substance or mixture 2.1 Classification of the substance or mixture GHS hazard class: Not classified as a hazardous chemical according to GHS 2.2 Label elements GHS pictogram Not classified as a hazardous chemical according to GHS 2.3 Other hazards: No applicable information.

| - 3.Composition/Information on Ingredients |  |  |
| --- | --- | --- |
| - 3.1 Product type： | Mixture |  |
| - 3.2 Ingredients |  |  |
| Chemical name | CAS NO. | Concentration % (w/w) |
| Aliphatic polyurethane polymer | Trade secret | 33±5 |
| Acetone | 67-64-1 | 0.5±0.5 |
| Acetic acid | 64-19-7 | 1.5±0.5 |
| Water | 7732-18-5 | 65±5 |

| - 4.First aid measures |  |
| --- | --- |
| - 4.1 General measures： | Immediately remove all contaminated clothing. |
| - 4.2 Ingestion： | Do not induce vomiting and seek medical advice. When unconscious, do not feed any food through the mouth. |
| - 4.3 Eye contact： | Hold the eyelids open and rinse thoroughly with lukewarm water for at least 10 minutes. Consult an ophthalmologist. |
| - 4.4 Skin contact： | Immediately wash with soap and plenty of water. If a skin reaction occurs, seek medical attention. |
| - 4.5 Inhalation： | If respiratory tract irritation occurs, seek medical attention. |

| - 5.Fire fighting measures |  |
| --- | --- |
| - 5.1 Suitable extinguishing - agents： | Carbon dioxide (CO₂), foam, or dry chemical powder. For large fires, use water spray. |
| - 5.2 Unsuitable extinguishing - agents： | High-volume water jet |
| - 5.3 Special hazards of - substances or mixtures： | Combustion releases carbon monoxide, carbon dioxide, nitrogen oxides, and small amounts of hydrogen cyanide. In the event of fire or explosion, do not breathe smoke or fumes. |
| - 5.4 Firefighting precautions - protective equipment： | Firefighters must wear self-contained breathing apparatus. Prevent contaminated fire-fighting water from entering soil, groundwater, or surface water. |

| - 6.Measures for accidental leakage |  |
| --- | --- |
| - 6.1 Firefighting precautions - protective equipment： | Wear protective equipment. Keep away from sources of ignition. Ensure adequate ventilation/exhaust. Keep unauthorized persons away. |
| - 6.2 Environmental - precautions： | Do not discharge into drains, wastewater, or soil. |
| - 6.3 Methods and material for containment and cleaning up： | Collect with a chemical absorbent or, if necessary, dry sand, and store in a sealed container. |

| - 7.Operation and storage |  |
| --- | --- |
| - 7.1 Precautions for safe handling： | Follow the usual precautions for handling chemicals. Avoid contact with skin and eyes. Keep away from food, drink, and tobacco. Wash hands before breaks and at the end of work. Store work clothing separately. Change contaminated or wet clothing. |
| - 7.2 Conditions for safe storage： | Keep the container tightly closed and store in a dry, well-ventilated place. To maintain product quality, follow the storage conditions specified in our product information sheet. |

| - 8.Exposure control / personal protection |  |
| --- | --- |
| 8.1 Exposure controls： |  |
| Respiratory protection： | Respiratory protective equipment is required during spraying. |
| Hand protection： | Protective gloves are recommended. |
| Suitable material for protective gloves： | EN 374-3 |
| Fluororubber – FKM: | Thickness ≥ 0.4 mm; breakthrough time ≥ 480 min. |
| Butyl rubber – IIR： | Thickness ≥ 0.5 mm; breakthrough time ≥ 480 min. |
| Nitrile rubber – NBR： | Thickness ≥ 0.35 mm; breakthrough time ≥ 480 min. |
| Recommendation: | Contaminated gloves should be discarded. |
| Eye protection： | Wear safety goggles/face shield. |
| Body protection： | Wear suitable protective clothing. |
| 8.2 Engineering controls： | No exposure limit information is available under EC Directive 2006/121/EC. |

| - 9.Physical and chemical properties |  |
| --- | --- |
| - 9.1 Appearance： | Translucent liquid |
| - 9.2 Odour threshold： | No data available. |
| - 9.3 pH value： | 4-6 |
| - 9.4 Ionicity： | Cationic |
| - 9.5 Initial boiling point： | Approx. 100 °C |
| - 9.6 Flash point： | No flash point below the initial boiling point |
| - 9.7 Evaporation rate： | Not applicable |
| - 9.8 Flammability (solid, gas): | Not applicable |
| - 9.9 Combustibility: | Not applicable |
| - 9.10 Relative vapour - density: | Not applicable |
| - 9.11 Density： | Not applicable |
| - 9.12 Water solubility: | Miscible |
| - 9.13 Surface tension: | No data available. |
| - 9.14 Logarithm of the octanol/wa - ter partition coefficient: | No data available. |
| - 9.15 Auto-ignition - temperature： | Not applicable |
| - 9.16 Ignition temperature： | No data available. |
| - 9.17 Dynamic viscosity： | < 1,000 mPa·s |
| - 9.18 Solid content： | 33±2% |
| - 9.19 Other information： | The above data are not product specifications. For product specifications, please refer to the product technical data sheet. |

| - 10.Stability and reactivity |  |
| --- | --- |
| - 10.1 Chemical stability： | No decomposition occurs when used as specified. |
| - 10.2 Hazardous decomposition products： | Flammable and harmful gases are generated during thermal decomposition. |
| - 10.3 Possible hazardous reactions： | No hazardous reactions occur when stored or handled properly. |

| - 11.Toxicity information |  |  |  |
| --- | --- | --- | --- |
| No toxicological studies are available for this product. |  |  |  |
| - 11.1 Acute toxicity： | Oral: | Polyurethane dispersion Median lethal dose (LD₅₀), rat: > 2,000 mg/kg Method: OECD Test Guideline 423 Toxicological study of a similar product |  |
|  | Inhalation: | Polyurethane dispersion Test atmosphere: dust/mist Assessment: This substance or mixture is not acutely toxic by inhalation. Method: OECD Test Guideline 403 Study of a similar product |  |
|  | Dermal: | Polyurethane dispersion Assessment: This substance or mixture is not acutely toxic via the dermal route. Study of a similar product |  |
| - 11.2 Subacute, subchronic, and delayed toxicity: | Polyurethane dispersion No data available. |  |  |
| - 11.3 Primary skin irritation: | Polyurethane dispersion Species: rabbit Result: Slight irritation Classification: Not irritating to skin Method: OECD Test Guideline 404 Study of a similar product |  |  |
| - 11.4 Primary mucous membrane irritation: | Polyurethane dispersion Species: rabbit Result: Slight irritation Classification: Not irritating to eyes Method: OECD Test Guideline 405 Study of a similar product |  |  |
| - 11.5 Sensitization： | Sensitisation Polyurethane dispersion Skin sensitisation according to the Buehler test (dermal): Species: guinea pig Result: Negative Classification: Not a skin sensitiser Method: OECD Test Guideline 406 Study of a similar product Skin sensitisation (local lymph node assay, LLNA) Species: mouse Result: Negative Classification: Not a skin sensitiser Method: OECD Test Guideline 429 Study of a similar product |  |  |
| - 11.6 In vitro genotoxicity: | Polyurethane dispersion Test type: Salmonella/microsome (Ames) test Metabolic activation: With/without Result: Negative Method: OECD Test Guideline 471 Study of a similar product Test type: In vitro chromosome aberration test Metabolic activation: With/without Result: Negative Method: OECD Test Guideline 473 Study of a similar product |  |  |
| - 11.7 n vivo genotoxicity: | Polyurethane dispersion No data available. |  |  |
| - 11.8 Carcinogenicity: | Carcinogenicity Polyurethane dispersion No data available. |  |  |
| - 11.9 Reproductive toxicity： | Fertility |  | Polyurethane dispersion No data available. |
|  | Developmental toxicity |  | Polyurethane dispersion No data available. |
| - 11.10 Specific target orga - n toxicity (single/repeate - d exposure): | STOT assessment – single exposure Polyurethane dispersion Based on the available data, the classification criteria are not met. STOT assessment – repeated exposure Polyurethane dispersion No data available. |  |  |
| - 11.11 Aspiration hazard： | Polyurethane dispersion No data available. |  |  |
| - 11.12 Additional information： | Polyurethane dispersion Carcinogenicity: No data available. Mutagenicity: Based on the available data, the classification criteria are not met. Teratogenicity: No data available. Reproductive toxicity/fertility: No data available. |  |  |

| - 12.Ecological information |  |
| --- | --- |
| - 12.1 Ecotoxicity： | No ecotoxicological studies are available for this product. Do not discharge into drains, wastewater, or soil. |
| - 12.2 Persistence and - degradability： | Biodegradability: < 60% in 28 days; this product is not readily biodegradable. Method: OECD Test Guideline 301D Study of a similar product |
| - 12.3 Other adverse effects： | No data available. |

| - 13.Handling precautions |  |
| --- | --- |
| Dispose of in accordance with applicable national standards and national or local regulations. Within the European Union, dispose of in accordance with the applicable provisions of the European Waste Catalogue (EWC). |  |
| Disposal methods： | Empty containers as completely as possible (e.g., by pouring, scraping, or draining until “drip-dry”). They may be sent to an appropriate collection point under existing chemical industry recycling schemes. Recycle containers in accordance with national regulations and applicable environmental requirements. Do not discharge waste via wastewater. |

| - 14.Transportation information |  |
| --- | --- |
| - 14.1 Road and rail transport： | Not dangerous goods for land transport under the ADG code. |
| - 14.2 Maritime transport： | Not dangerous goods for sea transport under the IMDG Code. |
| - 14.3 Air transport： | Not dangerous goods for air transport under IATA regulations. |
| - 14.4 Special precautions for user： | Not dangerous goods. |

- 15.Regulatory information

Safety, health, and environmental regulations specific to the substance or mixture

Other provisions：

Complies with the following regulations：

Regulations on the Safety Management of Hazardous Chemicals, State Council Decree No. 344

GB/T 16483-2008, Safety Data Sheet for Chemical Products: Content and Order of Sections

GB 13690-2009, General Rules for Classification and Hazard Communication of Chemicals

GB 20576–GB 20598, Safety Rules for Classification, Precautionary Labelling, and Precautionary Statements of Chemicals

GB 15258, General Rules for Preparation of Precautionary Labels for Chemicals

- 16.Other information

To the best of our knowledge, the information provided in this Safety Data Sheet is correct as of its date of issue. The information is intended only as guidance for the safe handling, use, production, storage, transport, disposal, and release of the specified material; it does not constitute a warranty or quality specification. This information applies only to the material specifically identified herein and may not be valid when that material is used in combination with other materials or in other processes, unless expressly stated in the text.